AMERICAN QUARTER MIDGET ASSOCIATION

SAFETY COMMITTEE

SAFETY CHECK PROCESS

RECOMMENDED:  2-MAN TEAM(s)

References:  QMA Rules and Administrative Guidelines  (Red Booklet)

WEIGHT OF CAR

160 Pounds Minimum Weight of Car 	Red Booklet - p.15 #24

DRIVER SAFETY EQUIPMENT

- Helmet	Red Booklet p. 18 #1

- Face Shield	Red Booklet p. 18#2

- Leather Jacket, or Lined Imitation Leather Jacket    	Red Booklet p. 18#3

- Leather Gloves  (Parapac or Nomex gloves are optional)    	Red Booklet p. 18 #3

- Neck Brace  	Red Booklet p. 19 #4

- Arm Restraints     	Red Booklet p. 14 #18B

- Safety Belt - Web-type with quick release buckle	Red Booklet p. 14 #18

- Shoulder Harness	Red Booklet p. 14 #18A

DRIVER POSITION IN CAR

- Brakes - Must lock wheel when depressed.       	Red Booklet p.11 #6

- Throttle -

- Steering Wheel - Hubs must be padded	Red Booklet p.15 #21

- Steering - Does not go past center (Rollover)	Red Booklet p.14 #20

- Switch - Off position is Back if switch is located on left outside
 of cowl.  Off position is Down if switch is located on center
 post of steering wheel.	Red Booklet p. 15 #22

- Body of car must cover driver’s legs.	Red Booklet p. 10 #3

- Must have 1” of clearance between top of helmet and cage.	Red Booklet p. 13 #17E

VISUAL INSPECTION OF CAR

- Carburetors will be within the tail section or covered.	Red Booklet p. 11 #4

- Breathers and Engine Vents will be under tail section.	Red Booklet p. 11 #4

- Engine Vents will be under tail section.	Red Booklet p. 11 #4

- Drive Chain must be covered .  (Unexposed)	Red Booklet p. 11 #9

- Exhaust System must extend to outside of tail section. However,
must be within the “triangle” of car, nerf, and outside tire edge.
Exhaust may not be positioned higher than rear tire.  Forward
 exhaust system must be fastened at outer end.	Red Booklet p. 11 #10

- Side Nerf Bars width must extend at least to center of rear tire,
but no further than outer edge of rear tire.	Red Booklet p. 12 #15

- Battery if mounted in cockpit must be enclosed and vented out.	Red Booklet p. 15 #30

- Weights must be bolted or welded to frame.	Red Booklet p. 15 #24

- Front-end of car must be enclosed. (screen or plate)	Red Booklet p. 10 #3

- Rods - Constructed of tubular aluminum.

- No Sharp Edges on car.  No Sharp Corners (square) on car. 	Red Booklet p. 10 #3

CAR WIDTH AND LENGTH

- Wheelbase:  42” - 56”  Overall Length:  84” max.	Red Booklet p. 15 #25

- Wheel Tread:  28” - 34” on center of tires.	Red Booklet p. 15 #26